LAB 2

Theory:

### Amplitude Scaling

C x(t) is a amplitude scaled version of x(t) whose amplitude is scaled by a factor C.

### Addition

Addition of two signals is nothing but addition of their corresponding amplitudes. This can be best explained by using the following example:

As seen from the diagram above,

-10 < t < -3 amplitude of z(t) = x1(t) + x2(t) = 0 + 2 = 2

-3 < t < 3 amplitude of z(t) = x1(t) + x2(t) = 1 + 2 = 3

3 < t < 10 amplitude of z(t) = x1(t) + x2(t) = 0 + 2 = 2

### Subtraction

subtraction of two signals is nothing but subtraction of their corresponding amplitudes. This can be best explained by the following example:

As seen from the diagram above,

-10 < t < -3 amplitude of z (t) = x1(t) - x2(t) = 0 - 2 = -2

-3 < t < 3 amplitude of z (t) = x1(t) - x2(t) = 1 - 2 = -1

3 < t < 10 amplitude of z (t) = x1(t) + x2(t) = 0 - 2 = -2

### Multiplication

Multiplication of two signals is nothing but multiplication of their corresponding amplitudes. This can be best explained by the following example:

As seen from the diagram above,

-10 < t < -3 amplitude of z (t) = x1(t) ×x2(t) = 0 ×2 = 0

-3 < t < 3 amplitude of z (t) = x1(t) ×x2(t) = 1 ×2 = 2

3 < t < 10 amplitude of z (t) = x1(t) × x2(t) = 0 × 2 = 0

The following operations can be performed with time:

### Time Shifting

x(t ±± t0) is time shifted version of the signal x(t).

x (t + t0) →→ negative shift

x (t - t0) →→ positive shift

### Time Scaling

x(At) is time scaled version of the signal x(t). where A is always positive.

|A| > 1 →→ Compression of the signal

|A| < 1 →→ Expansion of the signal

Note: u(at) = u(t) time scaling is not applicable for unit step function.

### Time Reversal

x(-t) is the time reversal of the signal x(t).

# Addition, Substraction and Scaling

clc;

figure(1);

x=[1 2 3 4];

y=[1 1 1 1];

t=0:1:3;

subplot(3,2,1);

stem(t,y);

xlabel("X");

ylabel("Y");

title("signal X [Anjan KC]");

subplot(3,2,2);

stem(t,x);

xlabel("X");

ylabel("Y");

title("signal Y [Anjan KC]");

z=x+y;

subplot(3,2,3);

stem(t,z);

xlabel("X");

ylabel("Y");

title("Signal X+Y [Anjan KC]");

z=x-y;

subplot(3,2,4);

stem(t,z);

xlabel("X");

ylabel("Y");

title("Signal X-Y [Anjan KC]");

a=input("Enter scaling factor:");

z=a*t;

subplot(3,2,5);

stem(z,x);

xlabel("X");

ylabel("Y");

title("Scaled Signal on x axis [Anjan KC]");

z=a*x;

subplot(3,2,6);

stem(t,z);

xlabel("X");

ylabel("Y");

title("Scaled Signal on y axis [Anjan KC]");

# Shifting

clc;

figure(2);

t=-2:1:2;

x=[-2 3 0 1 5];

n1=input("Enter delay amount");

n2=input("Enter advance amount");

subplot(3,2,1);

stem(t,x);

xlabel("X");

ylabel("Y");

title("input signal [Anjan KC]");

m=t+n1;

y=x;

subplot(3,2,2);

stem(m,y);

title("delayed signal [Anjan KC]");

k=t-n2;

z=x;

subplot(3,2,3);

stem(k,z);

title("advance signal [Anjan KC]")

# Folding

clc;

figure(3);

n=-1:2;

x=[3,-1,0,-4];

subplot(2,2,1);

stem(n,x);

title('Signal x(n)');

c=fliplr(x);

y=fliplr(-n);

subplot(2,2,2);

stem(y,c);

title('Folded Signal');