LAB 1

THEORY:

MATLAB is a numerical computing environment developed by MathWorks. MATLAB stands for “MATRIX LABORATORY”, and is a powerful computing environment for numerical calculations and multidimensional visualization.

MATLAB is popular for developing engineering applications because:

  - It reduces programming to data processing abstraction. Instead of focusing on intricate programming, it allows the user to focus on the theoretical concepts.

  - Developing code in MATLAB takes less time as compared to other programming languages.

  - It has a rich collection of library functions, called toolboxes, in almost every field of engineering. Users can access the library functions to build required applications.

  - It supports multidimensional visualization that allows experimental data to be rendered graphically in a comprehensible format.

  - MATLAB allows matrix manipulations, plotting of functions and data and implementation of algorithms.

Unit Impulse Signal : An ideal impulse signal is a signal that is zero everywhere but at the origin (t = 0), it is infinitely high.

Unit Step Signal: The step signal or step function is that type of standard signal which exists only for positive time and it is zero for negative time.

Ramp SIgnal: A ramp function or ramp signal is a type of standard signal which starts at 𝑡 = 0 and increases linearly with time.

Sine Wave Signal: A sine wave is a geometric waveform that oscillates (moves up, down, or side-to-side) periodically, and is defined by the function y = sin x.

CODE:

clc;

figure(1);

t=-5:0.1:5;

y=t==0;

subplot(2,2,1);

plot(t,y);

hold on;

stem(t,y);

xlabel("X");

ylabel("Y");

title("Unit Impulse Signal");

y=t>=0;

subplot(2,2,2);

plot(t,y);

hold on;

stem(t,y);

xlabel("X");

ylabel("Y");

title("Unit Step Signal");

y=t.*(t>=0);

subplot(2,2,3);

plot(t,y);

hold on;

stem(t,y);

xlabel("X");

ylabel("Y");

title("Unit Ramp Signal");

w=2*pi*5*t;

y=10*sin(w);

subplot(2,2,4);

plot(t,y);

hold on;

stem(t,y);

xlabel("X");

ylabel("Y");

title("Sine Signal");